Asbestos risk assessment

Material and priority assessment for one asbestos-containing material (ACM)

| Premises | ACM reference (from register) | Location / room |
| --- | --- | --- |
| Material / product | Assessed by | Date |

Part 1. Material assessment

How readily the material would release fibres if disturbed. Choose one score for each factor and add them up (total 2 to 12).

| Factor | Score | Examples (HSG227 Table 2) | Your score |
| --- | --- | --- | --- |
| Product type | 1 | Asbestos reinforced composites (plastics, resins, mastics, roofing felts, vinyl floor tiles, semi-rigid paints or decorative finishes, asbestos cement) |  |
|  | 2 | Asbestos insulating board, mill boards, other low density insulation boards, asbestos textiles, gaskets, ropes and woven textiles, asbestos paper and felt |  |
|  | 3 | Thermal insulation (e.g. pipe and boiler lagging), sprayed asbestos, loose asbestos, asbestos mattresses and packing |  |
| Extent of damage / deterioration | 0 | Good condition: no visible damage |  |
|  | 1 | Low damage: a few scratches or surface marks; broken edges on boards, tiles etc |  |
|  | 2 | Medium damage: significant breakage or several small areas where material has been damaged revealing loose asbestos fibres |  |
|  | 3 | High damage or delamination of materials, sprays and thermal insulation. Visible asbestos debris |  |
| Surface treatment | 0 | Composite materials containing asbestos: reinforced plastics, resins, vinyl tiles |  |
|  | 1 | Enclosed sprays and lagging, asbestos insulating board (exposed face painted or encapsulated), asbestos cement sheets etc |  |
|  | 2 | Unsealed asbestos insulating board, or encapsulated lagging and sprays |  |
|  | 3 | Unsealed laggings and sprays |  |
| Asbestos type | 1 | Chrysotile |  |
|  | 2 | Amphibole asbestos excluding crocidolite |  |
|  | 3 | Crocidolite |  |

| Material assessment total (2 to 12) | Band: 10+ high · 7-9 medium · 5-6 low · 4 or less very low |
| --- | --- |

Notes (sample or survey reference, photos taken, presumed or confirmed)

Part 2. Priority assessment

How likely the material is to be disturbed. Score each factor 0 to 3. If you use more than one factor under a heading, HSE says to average them, rounding up. Add the four heading scores together.

| Heading | Factor | Scores (HSG227 Table 3) | Factor score | Heading score |
| --- | --- | --- | --- | --- |
| Normal occupant activity | Main type of activity in area | 0 Rare disturbance (e.g. little used store room) · 1 Low disturbance (e.g. office type activity) · 2 Periodic disturbance (e.g. industrial or vehicular activity which may contact ACMs) · 3 High disturbance (e.g. fire door with AIB sheet in constant use) |  |  |
|  | Secondary activities for area | Score as above |  |  |
| Likelihood of disturbance | Location | 0 Outdoors · 1 Large rooms or well-ventilated areas · 2 Rooms up to 100 m² · 3 Confined spaces |  |  |
|  | Accessibility | 0 Usually inaccessible or unlikely to be disturbed · 1 Occasionally likely to be disturbed · 2 Easily disturbed · 3 Routinely disturbed |  |  |
|  | Extent / amount | 0 Small amounts or items (e.g. strings, gaskets) · 1 Up to 10 m² or 10 m pipe run · 2 Over 10 and up to 50 m² or pipe run · 3 Over 50 m² or 50 m pipe run |  |  |
| Human exposure potential | Number of occupants | 0 None · 1 1 to 3 · 2 4 to 10 · 3 More than 10 |  |  |
|  | Frequency of use of area | 0 Infrequent · 1 Monthly · 2 Weekly · 3 Daily |  |  |
|  | Average time area is in use | 0 Less than 1 hour · 1 1 to 3 hours · 2 3 to 6 hours · 3 More than 6 hours |  |  |
| Maintenance activity | Type of maintenance activity | 0 Minor disturbance (e.g. possible contact when gaining access) · 1 Low (e.g. changing light bulbs in AIB ceiling) · 2 Medium (e.g. lifting one or two AIB ceiling tiles to access a valve) · 3 High (e.g. removing a number of AIB ceiling tiles to replace a valve or for recabling) |  |  |
|  | Frequency of maintenance activity | 0 ACM unlikely to be disturbed for maintenance · 1 Once a year or less · 2 More than once a year · 3 More than once a month |  |  |

| Priority assessment total | Material total (from Part 1) | Overall risk score (material + priority) |
| --- | --- | --- |

Part 3. Decision

Action:  ☐ Leave in place, manage and monitor    ☐ Label    ☐ Repair, seal or enclose    ☐ Remove    ☐ Licensed contractor needed

| Action due by | Next condition check | Recorded in register on |
| --- | --- | --- |